Practical-1: Designing a Network Topology in Cisco Packet Tracer

Objective:

To design and configure a network topology in Cisco Packet Tracer that includes multiple switches and routers. The goal is to create a functioning network where devices can communicate across different segments.

Requirements:

- Network Devices:

  - 2 Routers

  - 3 Switches

  - 6 PCs (2 connected to each switch)

  - Appropriate cabling (Ethernet, Serial, etc.)

- IP Addressing:

  - Use the following subnets:

    - 192.168.1.0/24 for PCs connected to Switch 1

    - 192.168.2.0/24 for PCs connected to Switch 2

    - 192.168.3.0/24 for PCs connected to Switch 3

  - Assign IP addresses to all PCs and configure default gateways.

- Router Configuration:

  - Set up inter-router communication using a Static route, RIP and OSPF.

  - Configure the routers with appropriate interfaces and IP addresses.

  - Ensure that routing is functional between all subnets.

- Testing:

  - Ensure that all PCs can ping each other.

Submission:

Submit the following by:

- The Cisco Packet Tracer file of your network topology.

- A document containing the network diagram, IP addressing scheme, and configuration commands.